Installation Manual

Model – MG-ANT-6

Product Description

Indoor TV Antenna

Technical Specifications

*Frequency Range: VHF: 174-230MHz/UHF:470-862MHz

*Impedance: 75Ω

*Connector: F connector

Installation Steps

I. Antenna Assembly

1. Insert the square ring inside the corresponding socket, ensure full contact.

2. Place two bars in corresponding sockets, ensure tight fit

II. Installation

1. Connect to TV

(A) For TVs without built-in tuner

Attach the coaxial cable from the antenna to the ANT jack of your digital receiver

(B) For TVs with built-in tuner

Attach the coaxial cable from the antenna to the TV’s antenna input

- Set to antenna

Turn on TV, in your TV’S setup menu, set the tuner model to “antenna” or “Air”.

- Scan for channels

In your TV’s setup menu set the TV to “scan” or “auto scan for channels”.

Notice

To ensure you receive the maximum number of channels you may want to experiment re-positioning the antenna and re-scanning for channels.